Expt 7 Spectral Estimation

7.1 Power Spectral Density Estimation and Spectral Analysis of Noisy Sinusoidal Signals using Periodogram Method in MATLAB

Objectives:

1. To generate a discrete-time signal consisting of sinusoidal components and noise.

2. To estimate the Power Spectral Density (PSD) of the signal using the Periodogram method.

3. To identify the frequency components present in the signal.

4. To compare the signal power and noise power for assessing the quality of generate signal.

Code:

clc; clear; close all

fs=1000; t=(0:999)/fs;

s=sin(2*pi*100*t)+0.5*sin(2*pi*200*t);

n=0.3*randn(size(t));

x=s+n;

figure, plot(x), grid on

title('Signal with Noise')

figure, periodogram(x,[],[],fs)

title('PSD using Periodogram')

figure

X=abs(fft(x));

f=(0:length(X)-1)*fs/length(X);

plot(f(1:500),X(1:500)), grid on

title('Magnitude Spectrum')

figure

bar([mean(s.^2) var(n)])

set(gca,'XTickLabel',{'Signal','Noise'})

title('Power Comparison')

grid on

OUTPUT:

7.2 Analysis of Windowing Effects on Spectral Estimation Using Rectangular, Hamming, Hanning Windows in MATLAB

Objectives:

1. To estimate the spectrum of a signal using different window functions such as Rectangular, Hamming, and Hanning windows.

2. To analyse the effect of window selection on spectral leakage.

3. To compare the main-lobe width and side-lobe levels of different windows.

4. To evaluate the accuracy of frequency component detection.

CODE:

clc; clear; close all;

% Signal

fs = 1000; N = 256; t = (0:N-1)/fs;

x = sin(2*pi*100*t) + 0.8*sin(2*pi*110*t);

NFFT = 2048; f = (0:NFFT/2-1)*(fs/NFFT);

% Windows

w = {rectwin(N), hamming(N), hann(N)};

names = {'Rectangular','Hamming','Hanning'};

% Processing

for i = 1:3

xw = x(:).*w{i};

X = fft(xw,NFFT);

P{i} = abs(X(1:NFFT/2));

P{i} = P{i}/max(P{i});

end

%% Individual Spectrums

figure;

for i = 1:3

subplot(3,1,i);

plot(f,20*log10(P{i}),'LineWidth',1.5);

title(['Spectrum using ' names{i} ' Window']);

xlim([0 250]); grid on;

end

%% Comparison

figure;

plot(f,20*log10(P{1}),'k',f,20*log10(P{2}),'r',f,20*log10(P{3}),'b','LineWidth',1.5);

legend(names); title('Window Comparison'); xlim([50 170]); grid on;

%% Window Shapes

figure;

for i = 1:3

subplot(3,1,i); plot(w{i},'LineWidth',1.5);

title([names{i} ' Window']); grid on;

end

%% Peak Detection

for i = 1:3

[pks{i},loc{i}] = findpeaks(P{i},f,'MinPeakHeight',0.3);

fprintf('\n%s Window Frequencies:\n',names{i});

disp(loc{i});

end

%% Peak Marking (Hamming)

figure;

plot(f,20*log10(P{2}),'LineWidth',1.5); hold on;

plot(loc{2},20*log10(pks{2}),'ro','MarkerFaceColor','r');

title('Detected Frequencies (Hamming)'); xlim([50 170]); grid on;

%% Main Peaks

fprintf('\nMain Spectral Peaks:\n');

for i = 1:3

[~,idx] = max(P{i});

fprintf('%s Window Peak = %.2f Hz\n',names{i},f(idx));

end

OUTPUT:

Rectangular Window Frequencies:

100.0977  109.8633

Hamming Window Frequencies:

100.0977  109.8633

Hanning Window Frequencies:

100.0977  109.8633

Main Spectral Peaks:

Rectangular Window Peak = 100.10 Hz

Hamming Window Peak = 100.10 Hz

Hanning Window Peak = 100.10 Hz

RESULT:

The spectrum of the given signal was successfully estimated using Rectangular, Hamming, and Hanning window functions in MATLAB.