Expt 8 Multirate Signal Processing

8.1 Implementation and Analysis of Decimation and Interpolation in Multirate Signal Processing Using MATLAB

Objectives:

1. To generate a discrete-time sinusoidal sequence for multirate signal processing.

2. To perform decimation of a discrete-time sequence by a factor of 2.

3. To perform Interpolation of a discrete-time sequence by a factor of 2.

4. To analyze and compare the original, decimated, and interpolated sequences.

Code:

clc;

clear;

close all;

% Original Sequence

n = 0:40;

x = sin(0.3*pi*n);

% Decimation by 2

y = x(1:2:end);

nd = 0:length(y)-1;

% Interpolation by 2

v = zeros(1,2*length(x));

v(1:2:end) = x;

ni = 0:length(v)-1;

% Figure 1: Original Sequence

figure;

stem(n,x,'filled');

grid on;

title('Original Sequence x[n]');

xlabel('n');

ylabel('Amplitude');

% Figure 2: Decimated Sequence

figure;

stem(nd,y,'filled');

grid on;

title('Decimated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 3: Interpolated Sequence

figure;

stem(ni,v,'filled');

grid on;

title('Interpolated Sequence by 2');

xlabel('n');

ylabel('Amplitude');

% Figure 4: Comparison

figure;

stem(n,x,'b'); hold on;

stem(0:2:length(v)-2,v(1:2:end),'r');

legend('Original x[n]','Interpolated Samples');

grid on;

title('Comparison of Sequences');

xlabel('n');

ylabel('Amplitude');

Output:

8.2 Polyphase Implementation of Decimation and Interpolation Filters Using MATLAB

Objectives:

1. To perform sampling rate reduction of a discrete-time signal using a polyphase FIR decimation filter with a decimation factor of 2.

2.To perform sampling rate increase of a discrete-time signal using a polyphase FIR interpolation filter with an interpolation factor of 2.

3.To reconstruct the original signal from the decimated signal through interpolation and compare the reconstructed signal with the original input signal.

4.To demonstrate the computational efficiency and effectiveness of polyphase filter structures in multirate digital signal processing applications.

Code:

clc;

clear;

close all;

%% Input Signal

Fs = 1000;                    % Sampling Frequency

t = 0:1/Fs:1;

% Test Signal

%Time Vector

x = sin(2*pi*50*t) + 0.5*sin(2*pi*120*t);

%% FIR Low-Pass Filter Design

N = 20;                       % Filter Order

Fc = 0.4;

% Normalized Cutoff Frequency

h = fir1(N,Fc,hamming(N+1));

%% Decimation Factor

M = 2;

% Polyphase Decimation

y_dec = upfirdn(x,h,1,M);

%% Interpolation Factor

L = 2;

% Polyphase Interpolation

y_int = upfirdn(y_dec,h,L,1);

%% Adjust Length for Comparison

if length(y_int) > length(x)

y_int = y_int(1:length(x));

elseif length(y_int) < length(x)

x = x(1:length(y_int));

end

%% Time Axes

t1 = (0:length(x)-1)/Fs;

t2 = (0:length(y_dec)-1)/(Fs/M);

t3 = (0:length(y_int)-1)/Fs;

%% BEST GRAPH OUTPUT

figure('Name','Polyphase Decimation and Interpolation','NumberTitle','off');

% Original Signal

subplot(4,1,1)

plot(t1,x,'b','LineWidth',1.5)

title('Original Input Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Decimated Signal

subplot(4,1,2)

stem(t2,y_dec,'r','filled')

title('Decimated Signal (M = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Interpolated Signal

subplot(4,1,3)

plot(t3,y_int,'k','LineWidth',1.5)

title('Interpolated/Reconstructed Signal (L = 2)')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Comparison

subplot(4,1,4)

plot(t1,x,'b','LineWidth',1.5)

hold on

plot(t3,y_int,'r--','LineWidth',1.5)

title('Comparison of Original and Reconstructed Signals')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

%% Display Information

disp('--------------------------------------');

disp('POLYPHASE DECIMATION & INTERPOLATION');

disp('--------------------------------------');

fprintf('Original Signal Length = %d\n',length(x));

fprintf('Decimated Signal Length = %d\n',length(y_dec));

fprintf('Interpolated Signal Length  = %d\n',length(y_int));

%% Separate Figure for Better Visualization

figure;

plot(t1,x,'b','LineWidth',2)

hold on

plot(t3,y_int,'r--','LineWidth',2)

title('Original vs Reconstructed Signal')

xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal')

grid on

Output:

POLYPHASE DECIMATION & INTERPOLATION

Original Signal Length = 1001

Decimated Signal Length = 511

Interpolated Signal Length  = 1001

Result:

1. The input signal was successfully decimated by a factor of 2 using a polyphase FIR decimation filter.

2.The input signal was successfully interpolated by a factor of 2 using a polyphase FIR interpolation filter.

3. The original signal was reconstructed from the decimated signal and closely matched the input signal.

4.The polyphase filter structure reduced computational complexity and efficiently performed multirate signal processing operations.